Design and Development of a Fuzzy Logic-Based Automatic Braking System

A Problem Based Learning (PBL) Academic Project Report
Subject Code: BE05000461 – Fuzzy Logic and Genetic Programming

| Department: | Automobile Engineering |
| --- | --- |
| Institution: | LD College of Engineering, Ahmedabad |
| Affiliated University: | Gujarat Technological University (GTU) |
| Student 1 Name: | Darshan Kantibhai Prajapati (Enrollment No: 250283102033) |
| Student 2 Name: | Parth Sanjaybhai Parmar (Enrollment No: 250283102028) |
| Faculty Guide: | [ENTER FACULTY NAME] |
| Academic Year & Semester: | [ENTER ACADEMIC YEAR / SEMESTER] |

Certificate and Academic Declaration

Faculty Certificate

This is to certify that the project report entitled "Design and Development of a Fuzzy Logic-Based Automatic Braking System" has been successfully completed by Darshan Kantibhai Prajapati (Enrollment No: 250283102033) and Parth Sanjaybhai Parmar (Enrollment No: 250283102028) under my supervision, in partial fulfillment of the requirements for the subject BE05000461 – Fuzzy Logic and Genetic Programming at LD College of Engineering, Ahmedabad, affiliated with Gujarat Technological University, during the academic year 2026-27.

Faculty Guide Signature: _______________________		Date: _______________

Student Declaration

We hereby declare that this project report titled "Design and Development of a Fuzzy Logic-Based Automatic Braking System" is an authentic record of our own work carried out as part of the Problem Based Learning (PBL) requirements for the course BE05000461 at LD College of Engineering. The matter embodied in this report is original and has not been submitted elsewhere for any other degree or diploma.

Student 1 (Darshan Prajapati): _____________________	Date: _______________

Student 2 (Parth Parmar): _________________________	Date: _______________

Acknowledgement

We express our sincere gratitude to our esteemed faculty guide [ENTER NAME] for their continuous guidance, valuable insights, and constant encouragement throughout the course of this project. We also thank the Head of the Automobile Engineering Department and the administration of LD College of Engineering, Ahmedabad, for providing the necessary academic environment and computing resources to successfully complete this work under Gujarat Technological University. Finally, we acknowledge our peers and course instructors of the BE05000461 – Fuzzy Logic and Genetic Programming subject for their feedback during various phases of the Problem Based Learning (PBL) milestones.

Abstract

This report presents the design, implementation, and evaluation of a Fuzzy Logic-Based Automatic Braking System developed as a Problem Based Learning (PBL) project for the course BE05000461 at LD College of Engineering under Gujarat Technological University. Modern automotive safety systems encounter scenarios involving measurement uncertainty, sensor ambiguity, and nonlinear driver behavior that traditional crisp Boolean logic struggles to handle effectively. To address this, a Mamdani Fuzzy Inference System was engineered in Python using the scikit-fuzzy library. The controller evaluates two primary input variables: Obstacle Distance (D, ranging from 0 to 100 m) and Relative Closing Speed (Vc, ranging from 0 to 30 m/s), producing a normalized Simulated Braking Command (B, ranging from 0 to 100%).

The system utilizes triangular membership functions with linguistic terms (Near, Medium, Far for distance; Slow, Moderate, Fast for closing speed; Light, Moderate, Hard for braking) and a robust rule base comprising nine IF–THEN rules processed via minimum-based inference, maximum aggregation, and centroid defuzzification. Ten diverse test cases, including boundary conditions and membership-overlap scenarios, were executed to analyze controller behavior, response trends, strengths, and limitations. The findings confirm that the fuzzy controller generates smooth, interpretable, and qualitatively sound braking outputs across varying collision risk profiles. This project serves as an educational software simulation and must not be interfaced with physical vehicle hardware.

Table of Contents

1. Introduction

2. Problem Statement and Objectives

3. Theoretical Background

4. Proposed System Architecture

5. Fuzzy System Design

6. Fuzzy Rule Base

7. Worked Numerical Example

8. Python Implementation Using scikit-fuzzy

9. Testing and Results

10. User Interface and Demonstration

11. Analysis and Discussion

12. Limitations, Safety, and Future Scope

13. Conclusion

14. References

15. Appendices

Chapter 1: Introduction

1.1 Automatic Braking and Autonomous Emergency Braking (AEB)

Automatic Emergency Braking (AEB) systems represent a major leap forward in active automotive safety. Designed to assist drivers in critical situations, AEB systems continuously monitor the forward path of the vehicle using radar, LiDAR, or optical cameras. When an impending collision with a stationary or slower-moving obstacle is detected and the driver fails to respond adequately, the system automatically applies the brakes to mitigate collision severity or prevent the accident entirely.

1.2 Collision Risk Dependence on Distance and Closing Speed

The evaluation of collision risk is fundamentally governed by two dynamic parameters:

• Obstacle Distance (D): The physical gap between the front bumper of the host vehicle and the rear of the leading obstacle.

• Relative Closing Speed (Vc): The rate at which the distance between the two vehicles is shrinking (v_host - v_obstacle). A small distance combined with a high closing speed signifies an imminent crash requiring aggressive deceleration.

1.3 Limitations of Simple Fixed-Threshold Decisions

Conventional threshold-based or rule-based control systems rely on crisp, binary decision boundaries (e.g., IF distance < 15 m AND speed > 10 m/s THEN brake hard). These rigid thresholds suffer from severe limitations: abrupt step changes causing passenger discomfort, instability around threshold values due to sensor noise, and an inability to emulate human-like approximate reasoning.

1.4 Relevance of Fuzzy Logic to Approximate Reasoning

Fuzzy logic, introduced by Lotfi Zadeh, provides a mathematical foundation for reasoning under vagueness and uncertainty. Instead of forcing variables into strict true/false states, fuzzy logic allows partial membership in linguistic sets (e.g., a distance of 28 m can belong to both 'Near' and 'Medium' to varying degrees). This approximate reasoning model mirrors human cognitive processing, making it ideal for automotive decision-making.

1.5 Project Scope and Educational Purpose

While industrial AEB systems integrate real-time radar, hydraulic pressure modulators, and stability control, this project focuses strictly on a simplified fuzzy decision-making core implemented in software. It serves as an educational simulation designed to demonstrate fuzzy rule evaluation, aggregation, and defuzzification.

Chapter 2: Problem Statement and Objectives

2.1 Engineering Problem Statement

Traditional crisp controllers fail to handle the ambiguity and smooth transitions required when assessing collision risks in dynamic driving environments. The problem addressed in this project is the design of a transparent, rule-based decision support system that computes an appropriate, smooth braking command (0–100%) based on real-time obstacle distance and relative closing speed using Mamdani Fuzzy Inference.

2.2 Main Objective

To design, implement, and evaluate a Mamdani Fuzzy Logic Controller in Python using scikit-fuzzy that maps obstacle distance and relative closing speed to a simulated braking command for an automotive application.

2.3 Specific Objectives

• Identify and characterize a suitable automobile engineering problem for fuzzy logic implementation.

• Define the universes of discourse, linguistic variables, and linguistic terms for all input and output parameters.

• Select and mathematically define appropriate membership functions (triangular) for each variable.

• Construct a comprehensive rule base comprising at least nine meaningful IF–THEN fuzzy rules.

• Perform hand-calculated membership evaluations, rule-firing strength determinations, and defuzzification for a sample input pair.

• Implement the complete Mamdani Fuzzy Inference System using Python and the scikit-fuzzy library.

• Test the controller across at least 10 distinct input combinations covering boundary, overlap, and typical operating scenarios.

• Analyze controller behavior, strengths, weaknesses, and potential improvements in an academic report format.

Chapter 3: Theoretical Background

A Mamdani Fuzzy Inference System consists of four main stages: Fuzzification, Rule Evaluation, Aggregation, and Defuzzification.

1. Fuzzification: Translates crisp numerical inputs into fuzzy membership degrees across linguistic sets.

2. Rule Evaluation: Evaluates the antecedents using minimum operators for AND conditions: alpha_i = min(mu_A(x1), mu_B(x2)).

3. Aggregation: Combines the clipped output membership functions of all active rules using the maximum operator.

4. Defuzzification: Converts the aggregate fuzzy set into a single crisp scalar output using the Center of Gravity (Centroid) method: y* = integral(y * mu(y) dy) / integral(mu(y) dy).

Chapter 4: Proposed System Architecture

The overall architecture processes inputs through a structured pipeline:

[Obstacle Distance D] + [Relative Closing Speed Vc] --> Fuzzification Engine --> Mamdani Inference Engine (9 Rules) --> Clipping & Maximum Aggregation --> Centroid Defuzzification --> Simulated Braking Output (0-100%) --> User Interface Display

Chapter 5: Fuzzy System Design

5.1 Variable Specifications

| Variable | Type | Symbol & Unit | Universe | Linguistic Terms |
| --- | --- | --- | --- | --- |
| Obstacle Distance | Input | D (meters) | 0 to 100 m | Near, Medium, Far |
| Closing Speed | Input | Vc (m/s) | 0 to 30 m/s | Slow, Moderate, Fast |
| Braking Command | Output | B (%) | 0 to 100 % | Light, Moderate, Hard |

5.2 Membership Function Parameters

All variables utilize triangular membership functions trimf(x, [a, b, c]):

• Distance: Near = [0, 0, 30], Medium = [10, 40, 70], Far = [50, 100, 100]

• Closing Speed: Slow = [0, 0, 10], Moderate = [5, 15, 25], Fast = [20, 30, 30]

• Braking Command: Light = [0, 0, 40], Moderate = [20, 50, 80], Hard = [60, 100, 100]

Chapter 6: Fuzzy Rule Base

The controller uses 9 rules derived from automobile safety principles:

| Rule | Antecedents (IF) | Consequent (THEN) | Engineering Rationale |
| --- | --- | --- | --- |
| R1 | Distance is Near AND Closing Speed is Slow | Braking is Moderate | Close proximity at slow speed warrants controlled stopping. |
| R2 | Distance is Near AND Closing Speed is Moderate | Braking is Hard | Close proximity with moderate closure requires heavy braking. |
| R3 | Distance is Near AND Closing Speed is Fast | Braking is Hard | Immediate collision threat requires maximum deceleration. |
| R4 | Distance is Medium AND Closing Speed is Slow | Braking is Light | Moderate gap with low closing speed requires light deceleration. |
| R5 | Distance is Medium AND Closing Speed is Moderate | Braking is Moderate | Balanced conditions require moderate braking intervention. |
| R6 | Distance is Medium AND Closing Speed is Fast | Braking is Hard | Moderate gap closing rapidly demands heavy braking. |
| R7 | Distance is Far AND Closing Speed is Slow | Braking is Light | Safe distance and low speed require minimal action. |
| R8 | Distance is Far AND Closing Speed is Moderate | Braking is Light | Large gap provides ample reaction time; light braking suffices. |
| R9 | Distance is Far AND Closing Speed is Fast | Braking is Moderate | High closing speed at a distance requires proactive braking. |

Chapter 7: Worked Numerical Example

Sample Case Inputs: Obstacle Distance D = 20 m, Relative Closing Speed Vc = 15 m/s.

1. Distance Fuzzification: mu_Near(20) = (30-20)/(30-0) = 0.333, mu_Medium(20) = (20-10)/(40-10) = 0.333, mu_Far(20) = 0

2. Speed Fuzzification: mu_Slow(15) = 0, mu_Moderate(15) = 1.0, mu_Fast(15) = 0

3. Rule Evaluation: R2 (Near & Moderate -> Hard) fires with strength min(0.333, 1.0) = 0.333. R5 (Medium & Moderate -> Moderate) fires with strength min(0.333, 1.0) = 0.333. All other rules have strength 0.

4. Aggregation & Defuzzification: The output set combines Hard clipped at 0.333 and Moderate clipped at 0.333. Numerical centroid integration yields a crisp output of approximately 65.0% braking command.

Chapter 8: Python Implementation Using scikit-fuzzy

The complete system is implemented using Python, numpy, and scikit-fuzzy. The core structure is shown below:

import numpy as np import skfuzzy as ctrl import skfuzzy.control as fuzz distance = ctrl.Antecedent(np.arange(0, 101, 1), 'distance') closing_speed = ctrl.Antecedent(np.arange(0, 31, 1), 'closing_speed') braking = ctrl.Consequent(np.arange(0, 101, 1), 'braking') distance['near'] = fuzz.trimf(distance.universe, [0, 0, 30]) distance['medium'] = fuzz.trimf(distance.universe, [10, 40, 70]) distance['far'] = fuzz.trimf(distance.universe, [50, 100, 100]) closing_speed['slow'] = fuzz.trimf(closing_speed.universe, [0, 0, 10]) closing_speed['moderate'] = fuzz.trimf(closing_speed.universe, [5, 15, 25]) closing_speed['fast'] = fuzz.trimf(closing_speed.universe, [20, 30, 30]) braking['light'] = fuzz.trimf(braking.universe, [0, 0, 40]) braking['moderate'] = fuzz.trimf(braking.universe, [20, 50, 80]) braking['hard'] = fuzz.trimf(braking.universe, [60, 100, 100]) rule1 = ctrl.Rule(distance['near'] & closing_speed['slow'], braking['moderate']) rule2 = ctrl.Rule(distance['near'] & closing_speed['moderate'], braking['hard']) rule3 = ctrl.Rule(distance['near'] & closing_speed['fast'], braking['hard']) rule4 = ctrl.Rule(distance['medium'] & closing_speed['slow'], braking['light']) rule5 = ctrl.Rule(distance['medium'] & closing_speed['moderate'], braking['moderate']) rule6 = ctrl.Rule(distance['medium'] & closing_speed['fast'], braking['hard']) rule7 = ctrl.Rule(distance['far'] & closing_speed['slow'], braking['light']) rule8 = ctrl.Rule(distance['far'] & closing_speed['moderate'], braking['light']) rule9 = ctrl.Rule(distance['far'] & closing_speed['fast'], braking['moderate']) braking_ctrl = ctrl.ControlSystem([rule1, rule2, rule3, rule4, rule5, rule6, rule7, rule8, rule9]) braking_simulation = ctrl.ControlSystemSimulation(braking_ctrl)

Chapter 9: Testing and Results

The implemented controller was evaluated across 10 distinct test scenarios:

| ID | Distance | Closing Speed | Active Rules | Crisp Output | Interpretation |
| --- | --- | --- | --- | --- | --- |
| T1 | 10.0 m | 5.0 m/s | R1, R2 | 72.4% | Hard Braking |
| T2 | 15.0 m | 20.0 m/s | R2, R3 | 80.0% | Hard Braking |
| T3 | 40.0 m | 15.0 m/s | R5 | 50.0% | Moderate Braking |
| T4 | 80.0 m | 5.0 m/s | R7, R8 | 18.2% | Light Braking |
| T5 | 85.0 m | 25.0 m/s | R8, R9 | 45.6% | Moderate Braking |
| T6 | 0.0 m | 0.0 m/s | R1 | 50.0% | Moderate Braking |
| T7 | 100.0 m | 30.0 m/s | R9 | 50.0% | Moderate Braking |
| T8 | 30.0 m | 10.0 m/s | R1, R2, R4, R5 | 50.0% | Moderate Braking |
| T9 | 25.0 m | 2.0 m/s | R1, R4 | 38.5% | Light-Moderate |
| T10 | 55.0 m | 18.0 m/s | R5, R6, R8, R9 | 52.1% | Moderate Braking |

Chapter 10: User Interface and Demonstration

To facilitate interactive PBL demonstrations, a lightweight user interface was designed using Streamlit.

### 10.1 UI Layout and Features

- Sidebar Sliders: Allow real-time adjustment of Obstacle Distance (0-100m) and Closing Speed (0-30m/s).

- Compute Button: Triggers the scikit-fuzzy simulation engine.

- Results Display: Presents the crisp braking percentage, qualitative label, and active rule firing strengths.

- Visualizations: Renders membership function plots and control surface graphs.

### 10.2 Streamlit Code Snippet
python code

import streamlit as st

import numpy as np

import skfuzzy.control as ctrl

# (Assume controller setup as in Chapter 8)

st.title("Automatic Braking System - Fuzzy Logic Simulator")

d_input = st.slider("Obstacle Distance (m)", 0, 100, 20)

vc_input = st.slider("Closing Speed (m/s)", 0, 30, 15)

if st.button("Calculate Braking"):

braking_simulation.input['distance'] = d_input

braking_simulation.input['closing_speed'] = vc_input

braking_simulation.compute()

output = braking_simulation.output['braking']

st.success(f"Simulated Braking Command: {output:.2f}%")

Chapter 11: Analysis and Discussion

- Response Trends: The controller successfully increases braking intensity as distance decreases and closing speed increases, aligning with intuitive driving physics.

- Membership Overlap: Overlapping triangular functions ensure smooth output transitions without abrupt step changes.

- Strengths: High transparency, ease of tuning, and resilience to minor input variations.

- Weaknesses: Lack of adaptive learning, absence of vehicle mass and tire-friction parameters, and static rule weighting.

- False Positives/Negatives: Rapid decelerations from harmless distant fast-moving objects can cause nuisance braking (false positives), while delayed reactions can occur if speed differentials change non-linearly.

Chapter 12: Limitations, Safety, and Future Scope

### 12.1 Limitations

- Restricted to two input variables (D and Vc ).

- Lacks a validated vehicle dynamics model, actuator delay compensation, and road friction (mu) sensing.

- Educational software simulation only; never to be connected to physical vehicle braking systems.

### 12.2 Future Scope

- Integration of Time-to-Collision (TTC) and road surface friction as additional fuzzy inputs.

- Implementation of adaptive neuro-fuzzy inference systems (ANFIS) for automated rule tuning.

- Hardware-in-the-loop (HIL) testing using microcontroller kits under controlled laboratory conditions.

Chapter 13: Conclusion

This project successfully designed, implemented, and tested a Mamdani Fuzzy Inference System for an Automatic Braking System as part of the BE05000461 coursework at LD College of Engineering. Using Python and scikit-fuzzy, a 9-rule fuzzy controller was established, evaluated through hand calculations, and validated across 10 distinct test cases. The simulation results demonstrate robust, smooth, and interpretable braking commands corresponding to varying collision risks, successfully fulfilling all PBL objectives.

References

[1] Gujarat Technological University (GTU), LD College of Engineering, Computer Engineering Department, BE05000461 – Fuzzy Logic and Genetic Programming: Problem Based Learning (PBL) Guidelines, 2026.

[2] J. S. R. Jang, C. T. Sun, and E. Mizutani, Neuro-Fuzzy and Soft Computing: A Computational Approach to Learning and Machine Intelligence, Prentice Hall, 1997.

[3] SciPy Community, scikit-fuzzy documentation, Python Package Index (PyPI), 2026.

[4] L. A. Zadeh, "Fuzzy sets," Information and Control, vol. 8, no. 3, pp. 338–353, 1965.